OPEX per Energy Asset – Reference Document

Netherlands – AnyLogic Energy Model

# 1. Overview

This document describes the OPEX (operational expenditure) components for each energy asset. OPEX is split into fixed O&M (EUR/kW/year or EUR/vehicle/year) and variable O&M (EUR/kWh output). Fuel costs are excluded from OPEX and tracked separately as energy carrier costs.

Fixed O&M typically includes: scheduled maintenance and inspections, insurance premiums, land lease or site rental, monitoring and control systems, administrative overhead. Variable O&M includes: consumables (lubricants, filters, water treatment chemicals), unscheduled maintenance, and wear-related component replacement.

# Storage Electric (Li-ion BESS)

OPEX: Fixed: 8 EUR/kWh/yr; Variable: 0.5 EUR/MWh

Battery storage O&M is relatively low. Main costs are monitoring, thermal management, and eventual degradation/capacity loss. Stack augmentation or replacement is typically treated as a separate capital event.

# Photovoltaic (Solar PV)

OPEX: Fixed: 10 EUR/kW/yr

Solar PV has very low O&M requirements. Costs include panel cleaning, inverter monitoring/replacement, vegetation management, and insurance. Inverter replacement (~every 10–15 years) may be treated separately.

# Heat Pump Air (air-source)

OPEX: Fixed: 30 EUR/kW-th/yr

Includes annual maintenance contract (typical in NL: €150–200/year for residential), refrigerant checks, filter replacement, and insurance. Dutch maintenance contracts from Feenstra, Eneco Service typically cover all scheduled maintenance.

# Windmill (Onshore Wind)

OPEX: Fixed: 25 EUR/kW/yr

Wind O&M includes gearbox inspection, blade inspection/repair, generator maintenance, foundation monitoring, and insurance. Full-service contracts common in NL. IRENA reports significant O&M cost reduction over past decade.

# Electrolyser

OPEX: PEM: 35 EUR/kW/yr; Alkaline: 25 EUR/kW/yr

Excludes stack replacement (major overhaul every 60,000–80,000 operating hours = 30–40% of initial CAPEX). Includes water quality monitoring, gas quality checks, cooling system maintenance, safety inspections.

# Diesel/Gas Generators

OPEX: Diesel: 15 EUR/kW/yr + 10 EUR/MWh; Gas CHP: 30 EUR/kW/yr + 8 EUR/MWh

Rotating machinery has higher O&M than static assets. Includes oil changes, filter replacement, spark plug/injector maintenance, vibration monitoring, and emissions testing.

# Gas Burner (Boiler)

OPEX: Fixed: 3 EUR/kW-th/yr + 1 EUR/MWh variable

Gas boilers are very low maintenance. Annual burner tuning, safety valve testing, flue gas analysis, and boiler water treatment.

# Vehicles

OPEX: BEV car: 800; Van: 1,500; E-Truck: 8,000; H2-Truck: 10,000 EUR/vehicle/yr

Vehicle O&M covers scheduled maintenance (reduced for BEVs vs. ICE due to fewer moving parts), tire replacement, insurance, roadside assistance, and battery health monitoring. BEVs are typically 30–40% cheaper to maintain than diesel equivalents.

# Sources

IRENA – Renewable Power Generation Costs 2024

Danish Energy Agency – Technology Data

European Hydrogen Observatory

Lazard – Levelized Cost of Energy+

ANWB – Vehicle Running Costs NL

Rabobank – Battery Electric Commercial Vehicles NL

ICCT – Total Cost of Ownership